Mobile Home Tool and Material List

For Duct Sealing, Air Sealing, and Insulation Retrofits

| Duct Sealing |  |
| --- | --- |
| Materials | Tools |
| - Aluminum coil stock (16” width) - Duct mastic - Butyl-backed aluminum (6” to 12” wide) - 3/8” self-tapping hex head screws - Foil-backed insulation board - Fiberglass mesh tape - Silicone and latex caulk | - Latex gloves - Flashlight - Drop lights - Mirror - Shop Vac with HEPA filter - Paint brushes - Rags - Rubbing alcohol - Rechargeable drill and assorted bits - Metal bending brake (optional) - Standard WX hand toolkit - Garbage bags - Cleaning supplies |

| Belly Retrofit |  |
| --- | --- |
| Materials | Tools |
| - 1.5” drywall screws - 3” drywall screws - 3/8” self-tapping hex head screws - Foil-backed insulation board - House wrap (e.g., Typar) - Foil-backed insulation board - Belly Patch ( “Flexmend”) - Spray adhesive - Stitch staples stapler - Standard construction staples (9/16”) - Construction adhesive (e.g., Liquid Nails) - Lathe or furring strips - Foam sealant (one- and two-part foam) - Bubble foil - 6-mil plastic sheeting - Loose-fill fiberglass insulation | - Shop Vac - 3/8” rechargeable drill and assorted bits - 1/2'’ electric drill - 2-1/4” wood boring bit - Standard WX hand toolkit - Duct tape - Portable electric generator (5,000 watts) - Insulation machine - Standard flexible insulation hose (100’) - Gray PVC 2” pipe sections (8 pieces) - Extension cords (#12 gauge) - Stitch and standard staplers - Garbage bags - Cleaning supplies - Garden rake |

| Sidewall Retrofit |  |
| --- | --- |
| Materials | Tools |
| - 3/8” self-tapping hex head screws - RTV clear silicone caulk - R-13 high-density, unfaced fiberglass batts - 6-mil plastic sheeting | - Flash light - Mirror - 3/8” rechargeable drill and assorted bits - Standard WX hand toolkit - Long, flat pry bar - Insulation stuffing tool (e.g., Lexan or galvanized) - Garbage bags - Cleaning supplies |

| Roof Retrofit |  |
| --- | --- |
| Materials | Tools |
| Edge Lift Method - Loose-fill fiberglass insulation - 3/8” self-tapping hex head screws - 1.5” roofing nails - Mobile home gutter material - Putty tape - RTV silicone caulk - Elastomeric roof coating Top Fill Method - Loose-fill fiberglass insulation - 3/8” self-tapping hex head screws - RTV silicone caulk - High-temperature silicone caulk - Elastomeric roof coating - Roof patch such as “Peel ‘n Seal” - Galvanized steel patching material - Aluminum coil stock - Fiberglass mesh | - Standard WX hand toolkit - 3/8” rechargeable drill and assorted bits - Portable electric generator (5,000 watts) - Insulation machine - Standard flexible insulation hose (100’) - Gray PVC 2” pipe sections (8 pieces) - Extension cords (#12 gauge) - Clear acrylic 2” insulation fill tube (8’) - Ladders and/or scaffolding - Brushes for roof coating - Garbage bags - Cleaning supplies |