# Mobile Home Sidewall Retrofit

# Weatherization Installer/Technician Mobile Homes

Learning Objectives

By attending this session, participants will become aware of:

- How mobile home walls are constructed.

- Why mobile home walls account for a significant amount of heat loss.

- A cost-effective technique for re-insulating wall cavities.

- Specialized tools and materials for re-insulating wall cavities.

Key Terminology

Condensation

Conductive heat loss

High-density fiberglass

Weep holes

Supplemental Materials

Handouts & Resources

Greely, Kathy, John Randolph, and Bill Hill. “A Warm Wind Blows South: Virginia's Weatherization Evaluation.” Home Energy Jan./Feb. 1992: 15-21. www.homeenergy.org.

Loose-Fill Insulation Coverage Chart

Mobile Home Tool and Material List for Duct Sealing, Air Sealing and Insulation Retrofits

Scott, Bob, and Lyn M. Bartges. “Weatherizing Mobile Homes.” Home Energy July 2004. www.homeenergy.org

US DOE WAP, Midwest Regional Field Office. Midwest Weatherization Best Practices Field Guide May 2007: 186-187.

Classroom Props & Activities

- Sample of high-density fiberglass batt insulation.

- Specialized stuffing tool and other sidewall retrofit tools and materials such as:

  - Long prybar for removing staples in the wall cavity.

  - Strips of 6-mil plastic.

  - Hex-head screws.

If possible, use a scale model of a mobile home or video clips to reinforce the concept of siding removal and insulation of the wall cavity.

Class Overview

- Use the presentation, discussion, and handouts to introduce students to the key elements of accessing and insulating sidewalls:

  - Accessing the wall cavity.

  - Deciding whether additional insulation is justifiable.

  - Removing old insulation.

  - Inserting new insulation.

- Use props, demos, or video footage to reinforce technical aspects. Encourage questions and anecdotes from the field. Take breaks from the formal presentation to allow students to handle the tools and materials involved in the sidewall retrofit.