Cool Roof Post Evaluation Checklist

| PROJECT INFORMATION |  |
| --- | --- |
| Property Address: |  |
| Job# or Client ID: |  |
| Inspector Printed Name: |  |
| Inspector QCI Certification #: |  |
| Inspection Date: |  |
| Local code inspection(s): (if applicable) | Permit #_______________________________________Date Passed _________________ Permit #_______________________________________Date Passed _________________ |
| Project Roof Information: | Type (e.g., metal, shingle) _________ Age____ Pitch angle (if any) (e.g., 2:12, flat) ____________ |

| ENERGY AUDIT INFORMATION |  |
| --- | --- |
| Was an energy audit performed on this dwelling that included this installation? | ☐ Yes ☐ No |
| If yes, was the measure Savings to Investment (SIR) less than 1? | ☐ Yes ☐ No ☐N/A |

| PRODUCT INFORMATION |  |
| --- | --- |
| Manufacturer: |  |
| Product name: |  |
| Type (e.g., integrated or coating): |  |
| Manufacturer’s Solar Reflectance Index (SRI) or Solar Reflectance and Thermal Emittance: |  |
| COMMENTS: |  |

| INSTALLATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Yes | No | N/A |  | Note |
|  |  |  | Roof coating/integration installed per work order (e.g., complete coverage, specified material, etc.) |  |
|  |  |  | Cool roof meets minimum requirements set forth in SWS 5.0402.1 |  |
| COMMENTS: |  |  |  |  |

| PERFORMANCE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Yes | No | N/A |  | Note |
|  |  |  | Roof condition is expected to last as long as the manufacturer specified useful life of the product |  |
|  |  |  | Is the roof coating material rated by the Cool Roof Rating Council? (https://coolroofs.org/directory/roof) |  |
|  |  |  | Low sloping roof (9.5° or less, 2:12 rise over run) has a solar reflectance of at least 0.55 and a thermal emittance of at least 0.75 (or a solar reflectance index (SRI) of 64) |  |
|  |  |  | Steep sloping roof (9.5° or more, greater than a 2:12 rise over run) has solar reflectance of at least 0.20 and a thermal emittance of at least 0.75 (or a solar reflectance index (SRI) of 16) |  |
| COMMENTS: |  |  |  |  |

| CLIENT EDUCATION |  |  |  |  |
| --- | --- | --- | --- | --- |
| Yes | No | N/A |  | Note |
|  |  |  | Occupant(s), owner(s), and/or maintenance staff have been provided with user’s manual, warranty information, installation instructions, and installer contact information |  |
| COMMENTS: |  |  |  |  |

ADDITIONAL COMMENTS

______________________________________________________________________    _______________________________________

Signature of Inspector											Date